Critical-Phase Shadow Reservoirs (CPSR / PCSR)

Essential Theory & Algorithms, with the Google-Willow → IBM-Qiskit Port Modifications

Tunable Quantum Reservoirs with Low-Inference-Overhead Classical-Shadow Readout (LUQPI framework)

Scope. This document collects the essential theory behind the CPSR/PCSR quantum-reservoir-computing project and every algorithm it uses, written as backend-agnostic pseudocode alongside the concrete modifications made when porting the reference implementation from Google Willow / Cirq to IBM Quantum / Qiskit. It is organized as: (I) the physics and learning theory; (II) the theory of the hardware port; (III) the algorithms with their modifications; and (IV) consolidated modification and results tables. Symbols follow the code: N reservoir qubits, g the critical-phase parameter, K shadow shots.

# Part I — Essential Theory

## 1. Quantum Reservoir Computing (QRC)

Reservoir computing offloads temporal computation onto a fixed, high-dimensional nonlinear dynamical system — the reservoir — and trains only a lightweight linear/shallow readout on the reservoir’s state features. A quantum reservoir uses the exponentially large Hilbert space of N qubits (dimension 2N) as the feature space. At each timestep the scalar input ut is encoded into single-qubit rotations, a fixed entangling unitary U scrambles the state, and a set of observables is read out to form the feature vector xt. Only the classical readout is learned; the quantum map is never trained.

Reservoir map. With an input-encoding unitary V(u) and a fixed reservoir unitary U, the state evolves as:

|ψₜ⟩ = U · V(windowₜ) |0⟩⊗ᴺ ,   xₜ = ⟨ψₜ| { Pᵢ , PᵢPⱼ } |ψₜ⟩

The features xt are expectation values of 1- and 2-body Pauli operators. The readout ŷₜ = f(xₜ) is a Ridge regressor or a small MLP.

## 2. Input Encoding & the Fixed Reservoir Layer

Inputs are fed through a sliding window of length W (default 8). Each slot in the window is mapped to a qubit (round-robin) and to a phase weight that grows across the window, so more recent inputs act more strongly — this gives the reservoir its fading memory. The per-qubit rotation angle is:

θᵢ = Σ_w  π · u_w · s_w   with   s_w = linspace(0.5, 1.0, W),   slot w → qubit (w mod N)

The encoded rotations are applied as Ry(θᵢ) on each qubit, followed by the fixed reservoir step U (entangling layer + fixed single-qubit biases), repeated reps times. The biases (bias_z, bias_x) are drawn once from a fixed seed and never trained.

## 3. The Critical Phase and the Edge of Chaos

The single most important knob is the entangling strength g. Each native two-qubit gate is raised to a fractional power to interpolate continuously between no entanglement and a full entangler:

CZ^(2g/π)  ≡  CP(2g)   :   g = 0 → Identity (ordered) ,   g = π/2 → full CZ (chaotic)

Sweeping g from 0 to π/2 drives the reservoir through three regimes — ordered, critical (edge of chaos), and chaotic. Computational power is maximized at the critical point, where the reservoir is maximally sensitive to inputs while still retaining memory. Three diagnostics locate it:

### 3.1 Half-system entanglement entropy

Bipartition the N qubits into halves; the von Neumann entropy of the reduced state measures how much the reservoir spreads information. Computed from the singular values of the reshaped state vector:

S_half = − Σ λᵢ² log λᵢ² ,   λ = svd( reshape(ψ, 2^⌊N/2⌋, 2^⌈N/2⌉) )

### 3.2 Operator entanglement (scrambling power)

A basis-independent measure of the reservoir unitary itself. Reshape U into its operator-Schmidt form across the bipartition and take the entropy of the normalized squared singular values. This peaks at the edge of chaos and is the primary criticality signal; the measured peak (≈ 1.4) coincides with the task-error minimum.

S_op = − Σ pᵢ log pᵢ ,   pᵢ = σᵢ² / Σⱼ σⱼ²   from the reshuffled U

### 3.3 Spectral form factor

A complementary chaos probe: SFF = |Tr U|² / d, with d = 2N. Small values indicate spectral rigidity characteristic of the chaotic regime.

## 4. Classical-Shadow Readout

Reading out all 1- and 2-body Pauli expectations directly would require many separate measurement settings. Classical shadows estimate all of them at once from randomized single-qubit measurements. For each shot, pick a random Pauli basis bᵢ ∈ {X, Y, Z} per qubit, rotate into it, and measure in the computational basis. An unbiased estimator of any matched Pauli string of weight w is:

⟨P⟩ ≈ (1/K) Σ_shots  3^w · ∏_{i∈supp} (−1)^{bitᵢ}   (only shots whose basis matches P contribute)

The 3w factor is the inverse of the depolarizing channel from random single-qubit bases. Two estimators appear in the code: an exact-state shortcut (compute the true Pauli expectation, then add variance-correct Gaussian shot noise of standard deviation √(3w/K)) for fast simulation, and the faithful randomized-measurement estimator above for hardware. Performance saturates around K ≈ 1000 shots per feature.

## 5. LUQPI — Learning Under Quantum Privileged Information

The central idea that makes CPSR practically valuable. The quantum reservoir is treated as privileged information available only at training time. A purely classical model is trained on the shadow features; at deployment it runs on classical inputs alone, with no QPU in the inference loop. This yields two benefits:

- Sample efficiency: shadow features lower prediction error at small training-set sizes; the classical-only baseline only catches up once enough data is available (the LUQPI crossover, near N_train ≈ 150).

- Inference speed: because the QPU is absent at deployment, CPSR inference latency equals a classical model’s (≈ 0.1 ms/pred) versus ≈ 60 ms/pred for standard QRC that must query hardware per prediction.

Honest scope. This is a sample-efficiency separation at small training sets and a deployment-latency advantage — not an asymptotic exponential quantum speedup. The commercial value is fast, noise-tolerant deployment.

# Part II — Theory of the Willow → IBM Port

## 6. Native-Gate Correspondence

Google Willow and IBM Heron both expose a native controlled-Z, so the criticality knob transfers exactly. Cirq’s cirq.CZ ** (2g/π) equals Qiskit’s controlled-phase qc.cp(2g, a, b): at g=0 it is the identity, at g=π/2 it is a full CZ. IBM Heron’s basis is {cz, rz, sx, x, id}; every CP(2g) is transpiled into that native set. No SWAPs are introduced because all reservoir edges are chosen to be real device couplers.

## 7. Endianness (a correctness-critical detail)

The numpy reservoir math is big-endian (qubit 0 = most-significant bit, left-most in the Kronecker product). Qiskit’s Operator is little-endian. Extracting the unitary as Operator(qc.reverse_bits()).data yields a big-endian matrix, so the entire downstream numpy stack (Pauli features, entropies, operator entanglement, reservoir evolution) runs unchanged. For measured bitstrings, Qiskit returns little-endian strings, so reservoir qubit i is recovered as bitstring[::-1][i].

## 8. The Heavy-Hex Topology Problem — and its Solution

Willow is a square grid whose smallest cycle is a 4-qubit plaquette; the original code distinguished reservoir topologies by how many such short cycles (edges) they contained. IBM Heron is a heavy-hex lattice with two properties that break that approach:

- Girth 12: there are no small cycles, so at N = 6 every connected subgraph is a tree (5 edges). Topologies cannot differ by edge count.

- Degree-3 “hub” vertices are never adjacent — they are always separated by degree-2 bridge qubits.

Solution: distinguish topologies by graph structure (branching / number of hubs) rather than edge count. This produces a monotone ladder of scrambling power at g/π = 0.30, matching the paper’s reported peak (≈ 1.48):

| Topology | Structure | S_op @ g/π=0.30 |
| --- | --- | --- |
| Disconnected | No edges (product state) | 0.00 |
| Zigzag | Plain path, degree ≤ 2 | 0.74 |
| Chain | Path routed through one degree-3 hub | 1.41 |
| Ladder | Two degree-3 hubs bridged via a degree-2 qubit | 1.53 |

The closed-loop motif for correlation spreading — Willow’s 4-cycle plaquette — becomes a heavy-hex hexagonal ring (12-cycle), found via a cycle basis of the coupling graph and used for the device-overlay figure. All topologies remain 100% native with zero SWAPs.

## 9. Noisy Validation: QVM → AerSimulator.from_backend

Cirq’s Willow Pink Quantum Virtual Machine is replaced by AerSimulator.from_backend(FakeTorino()), a calibrated noisy simulator carrying the device’s measured T₁/T₂, gate, and readout errors — the direct IBM analog of the QVM. Setting USE_REAL_HARDWARE = True routes identical circuits to a real QPU through SamplerV2. The measured shadow-noise budget on IBM (mean |Δ⟨Z⟩| ≈ 0.33, shot floor 0.173, hardware excess ≈ 0.15) reproduces the paper’s Willow figures almost exactly.

# Part III — Algorithms (with Port Modifications)

Each algorithm is given as pseudocode followed by a Modified for IBM note describing what changed from the Cirq/Willow original. Line references use the code identifiers.

## Algorithm 1 — Device Backend & Coupler Graph

Pseudocode

get_ibm_backend():

if USE_REAL_HARDWARE:

service = QiskitRuntimeService()

return service.backend(NAME) or service.least_busy(...)

else:

return FakeTorino()          # 133q heavy-hex, native CZ

ibm_couplers(): return sorted, de-duplicated undirected edges

of backend.coupling_map            # native CZ pairs

Modified for IBM: replaces create_default_noisy_quantum_virtual_machine(‘willow_pink’) and Willow’s grid qubits. Couplers now come from the Heron coupling map; a NetworkX graph of these edges backs all topology construction.

## Algorithm 2 — Native Topology Builders

All builders select real device qubits by lowest readout error and use only native couplers (zero SWAPs).

Pseudocode

_pick_path(N):      DFS a simple path, degree ≤ 2 (relax to 3 if forced)

_pick_single_hub(N): grow a path through ONE degree-3 vertex (1 hub)

_pick_double_hub(N): find two degree-3 hubs at graph-distance 2,

join them through their shared degree-2 bridge

_hex_ring(≥12):     pick lowest-error closed cycle from cycle_basis(G)

TOPOLOGY_BUILDERS = { Disconnected: product_state,

Zigzag: brick_wall(_pick_path),

StarHub: star_hub, Chain: muta_chain(_pick_single_hub),

Ladder: muta_ladder(_pick_double_hub) }

Modified for IBM: entirely new. The Willow version distinguished topologies by counting 4-cycles/edges; on heavy-hex (girth 12, non-adjacent hubs) topologies are instead distinguished by branching structure, yielding the monotone S_op ladder in §8. The 4-cycle plaquette motif becomes the 12-cycle hex ring.

## Algorithm 3 — Critical Reservoir Unitary

Pseudocode

critical_reservoir_unitary(topo, g, bias_z, bias_x):

qc = QuantumCircuit(N)

for layer in pack_moments(topo.edges):     # parallel CZ layers

for (a,b) in layer: qc.cp(2*g, a, b)   # CZ^(2g/π) = CP(2g)

for i in range(N): qc.rz(bias_z[i], i)     # fixed biases

for i in range(N): qc.rx(bias_x[i], i)

return Operator(qc.reverse_bits()).data    # big-endian unitary

Modified for IBM: cirq.CZ**(2g/π) → qc.cp(2g); the unitary is extracted through reverse_bits() to preserve the big-endian convention. pack_moments greedily groups disjoint edges into parallel layers (the Qiskit analog of Cirq moments).

## Algorithm 4 — PCSR Reservoir Run (state evolution)

Pseudocode

pcsr_reservoir(topo, u_seq, g, bias_z, bias_x, W=8, reps=2):

U_step = matrix_power(critical_reservoir_unitary(...), reps)

for t in range(T):

window = last W inputs ending at t   (zero-padded at start)

θᵢ = Σ_w π·u_w·s_w  for slot→qubit (w mod N)

V = ⊗ᵢ Ry(θᵢ)                    # input encoding

ψₜ = (U_step @ V) @ |0⟩

return states (T × 2^N)

Modified for IBM: algorithmically unchanged — it consumes the big-endian U_step from Algorithm 3, so the Kronecker-product encoding and evolution work verbatim. Feasible for N ≲ 8 (state-vector sized), consistent with the paper’s N = 8 ceiling.

## Algorithm 5 — Criticality Metrics

Pseudocode

half_system_entropy(ψ, N):

λ = svd( reshape(ψ, 2^⌊N/2⌋, 2^⌈N/2⌉) )

return −Σ λ² log λ²

operator_entanglement(U, N):

reshuffle U → (dh² × dh²);  p = svd² / Σsvd²

return −Σ p log p          # peaks at edge of chaos

spectral_form_factor(U): return |Tr U|² / d

Modified for IBM: unchanged (pure linear algebra on the state/unitary). Only the input unitary’s endianness matters, which Algorithm 3 already fixes.

## Algorithm 6 — Shadow Feature Extraction

Pseudocode

# Fast simulation estimator

exact_pauli_expectations(ψ, N, max_weight=2):

compute ⟨Zᵢ⟩,⟨Xᵢ⟩,⟨Yᵢ⟩ and matched 2-body ⟨PᵢPⱼ⟩ exactly

shadow_features(states, N, K):

v = exact_pauli_expectations(ψₜ)

if K>0: v += √(3^w / K) · randn()     # variance-correct noise

# Faithful hardware estimator

randomized_measurement_features(ψ, N, K):

repeat K times: pick basis bᵢ∈{X,Y,Z}; rotate; sample outcome

for each matched Pauli: est += 3^w · ∏(−1)^{bit}

return est / K

Modified for IBM: the estimators are identical in form; on hardware the outcome sampling is done by the Qiskit SamplerV2 (Algorithm 9) instead of a numpy multinomial, and bit order is corrected via bitstring[::-1].

## Algorithm 7 — LUQPI Training & Evaluation

Pseudocode

luqpi_train_and_eval(X_shadow, y, model=‘mlp’):

tr, te = interleaved_split(T, train_frac, washout)

model = MLP(64,32) or Ridge(α);  model.fit(X_shadow[tr], y[tr])

return NRMSE( model.predict(X_shadow[te]), y[te] )

classical_only_baseline(u, y, lags=10):  # no quantum features

X = time-delay embedding of u;  fit same model;  return NRMSE

Modified for IBM: unchanged — fully classical (scikit-learn). This is the whole point of LUQPI: no QPU at inference, so this stage is backend-independent.

## Algorithm 8 — Phase-Criticality Scan

Pseudocode

phase_scan(N, T, g ∈ [0, π/2]):

for each g:

U = critical_reservoir_unitary(Chain, g, ...)

record S_op(U), S_half(ψ), memory_capacity, task NRMSE

→ locate g* = argmin NRMSE  (≈ edge-of-chaos S_op peak)

Modified for IBM: runs on the Chain (single-hub) heavy-hex topology; titles/labels report “IBM Heron”. Reproduced results: S_op peak 1.41 and k-Pauli NRMSE minimum both at g/π = 0.30.

## Algorithm 9 — IBM Hardware / Noisy Validation

Pseudocode

build_pcsr_circuit(topo, window, g, biases, basis):

Ry encode; for layer: cp(2g); rz,rx biases;

apply basis rotation per qubit (H for X, S†H for Y);

measure res_q[i] → classical bit i;

transpile(qc, backend, opt_level=1)      # → native {cz,rz,sx,x}

run_ibm_pcsr(topo, T, g, n_bases, shots):

sampler = SamplerV2( AerSimulator.from_backend(FakeTorino) )

for t, for each random basis: collect bitstrings

reconstruct_shadow_pauli(samples, bases):

⟨Pᵢ⟩ = 3·mean(±1) ; ⟨PᵢPⱼ⟩ = 9·mean(±1) on matched shots

Modified for IBM: complete rewrite of the Cirq QVM path. get_qvm_sampler → SamplerV2 over a noisy Aer backend (or a real QPU); circuits are transpiled to the native gateset; measurement bit i maps to reservoir qubit i and is recovered with little-endian correction. Only terminal measurements are used (no mid-circuit measurement or feedforward).

# Part IV — Consolidated Modifications & Results

## 10. Cirq/Willow → Qiskit/IBM Modification Summary

| Concern | Original (Cirq / Willow) | Port (Qiskit / IBM Heron) |
| --- | --- | --- |
| Device | Willow Pink QVM (105q, square grid) | FakeTorino / Heron (133q, heavy-hex) |
| Critical gate | cirq.CZ ** (2g/π) | qc.cp(2g, a, b) |
| Native basis | GoogleCZTargetGateset | {cz, rz, sx, x, id} via transpile |
| Unitary endianness | big-endian (native) | Operator(qc.reverse_bits()).data |
| Bit readout | result.measurements['m'] | bitstrings[::-1][i] |
| Topology basis | 4-cycle plaquettes / edge count | branching structure (hubs), 12-cycle ring |
| Closed-loop motif | 4-cycle plaquette | hexagonal ring (12-cycle) |
| Noisy validation | Willow Pink QVM | AerSimulator.from_backend(FakeTorino) |
| Real-HW path | cirq-google engine | QiskitRuntimeService + SamplerV2 |
| Overlay coords | q.row / q.col grid | networkx kamada-kawai layout |
| Inference latency est. | 300 shots @ 63 kHz → 4.76 ms | 300 shots @ 5 kHz → 60 ms |

## 11. Measured Findings on IBM Heron

| Quantity | Value | Meaning |
| --- | --- | --- |
| Edge-of-chaos g*/π | 0.30 | NRMSE minimum aligns with S_op peak |
| Operator-entanglement peak | ≈ 1.41 | Max scrambling (hub topology); paper ≈ 1.48 |
| LUQPI crossover | N_train ≈ 150 | Classical catches up only with more data |
| CPSR inference latency | ≈ 0.1 ms/pred | Matches classical-only |
| Standard-QRC latency | ≈ 60 ms/pred | > 600× slower (QPU per call) |
| Shadow shots to saturate | K ≈ 1000 | Well within in-session budget |
| IBM noise contribution | ≈ 0.15 mean ¦Δ⟨Z⟩¦ | Comparable to the LUQPI margin |

Reproducibility. All results were produced by executing the ported notebook end-to-end on the calibrated noisy Heron backend. To run on real hardware, set USE_REAL_HARDWARE = True and IBM_BACKEND_NAME (requires a saved QiskitRuntimeService account); every circuit and the shadow-reconstruction path are backend-agnostic.